Setup av Router-01. Switch-01, Switch-02, subnet /26 och VLAN 10, 20,30,99

1. Definiera alla nätverk

1. Kontor – vlan 10

2. Ekonomi – vlan 20

3. Gast – vlan 30

4. Drift – vlan 99

Kontor Network: 192.168.1.0, Router: 192.168.1.1

Ekonomi Network: 192.168.1.64, Router: 192.168.1.65

Gast Network: 192.168.1.128, Router: 192.168.1.129

Drift network: 192.168.1.192, Router: 192.168.1.193

Varje network vill ha 10 reserverad address till statik enheter.

Kontor Network excluded-address 192.168.1.2 192.168.1.11

Ekonomi Network excluded-address 192.168.1.66 192.168.1.75

Gast Network excluded-address 192.168.1.130 192.168.1.139

Drift network excluded-address 192.168.1.194 192.168.1.203

1. Router setup

Byta Hostname och bestämmer vlans på interface

Enable

conf t

Hostname RT-01-Attila

int g0/0/0

no shutdown

end

write

Fix alla vlan på router G0/0/0 port

int g0/0/0

no ip address

int g0/0/0.10

encapsulation dot1q 10

ip address 192.168.1.1 255.255.255.192

int g0/0/0.20

encapsulation dot1q 20

ip address 192.168.1.65 255.255.255.192

int g0/0/0.30

encapsulation dot1q 30

192.168.1.129 255.255.255.192

int g0/0/0.99

encapsulation dot1q 99

192.168.1.193 255.255.255.192

ip dhcp excluded-address 192.168.1.2 192.168.1.11

ip dhcp excluded-address 192.168.1.66 192.168.1.75

ip dhcp excluded-address 192.168.1.130 192.168.1.139

ip dhcp excluded-address 192.168.1.194 192.168.1.203

ip dhcp pool Kontor

192.168.1.0 255.255.255.192

default-router 192.168.1.1

dns-server 8.8.8.8

ip dhcp pool Ekonomi

192.168.1.64 255.255.255.192

default-router 192.168.1.65

dns-server 8.8.8.8

ip dhcp pool Gast

192.168.1.128 255.255.255.192

default-router 192.168.1.129

dns-server 8.8.8.8

ip dhcp pool Drift

192.168.1.192 255.255.255.192

default-router 192.168.1.193

dns-server 8.8.8.8

2. switch 0 setup

enable

conf t

Hostname SW-01-Attila

vlan 10

name KONTOR

vlan 20

name EKONOMI

vlan 30

name GAST

vlan 99

name DRIFT

int G0/1 switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

switchport native vlan 999

int g0/2 switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

switchport native vlan 999

PC 6 - DRIFT

int f0/1

switchport mode access

switchport access vlan 99

PC7 -DRIFT

int f0/2

switchport mode access

switchport access vlan 99

3. Switch 2 setup

enable

conf t

Hostname SW-01-Attila

vlan 10

name KONTOR

vlan 20

name EKONOMI

vlan 30

name GAST

vlan 99

name DRIFT

Pc 0 och Pc 1 – VLAN 10 - KONTOR

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 10

Pc 2 och Pc 3  - VLAN 20 – EKONOMI

interface fa0/3

switchport mode access

switchport access vlan 20

Interface fa0/4

switchport mode access

switchport access vlan 20

Pc 4 och Pc 5 – VLAN 30 – GAST

int fa0/5

switchport mode access

switchport access vlan 30

int fa0/6

switchport mode access

switchport access vlan 30

Check on both switch om vlans and trunk ports are working.

Show vlan brief

show interfaces trunk